SAN PATRICIO COUNTY

Commercial Development Review Process — Floodplain & Storm Water Management

## Project Information

| Project / Development Name: |  | Applicant / Developer: |  |
| --- | --- | --- | --- |
| Engineer of Record (if known): |  | Parcel ID(s) / Address: |  |
| Acreage: |  | FEMA FIRM Panel Number: |  |
| Date Submitted: |  | Contact Phone / Email: |  |

## Review Steps

|  | Step | Action | Responsible Party |
| --- | --- | --- | --- |
| ☐ | 1 | Pre-Application Phase — File Property Deed • File the property deed with the County Clerk to be filed. | Developer / County Clerk |
| ☐ | 2 | Platting / Property ID Number (PI#) • Take the filed deed to the Appraisal District for platting. • Receive a PI# (Property ID Number) for the parcel. | Developer / Appraisal District |
| ☐ | 3 | Address Verification (GIS/NG911) & Storm Water Management Submittal • Take the property to GIS/NG911 to apply for a new address or verify the existing address on the property. • Submit the site grading plan and site drainage plan for storm water management to the San Patricio Drainage District at mrvanecek@sbcglobal.net, with the Floodplain Manager copied on the email. | Developer / GIS-NG911 / Drainage District |
| ☐ | 4 | Floodplain Development Permit Application • Required if any work (grading, fill, structures) occurs within the mapped floodplain. • No-Rise Certification required for floodway impacts; a CLOMR/LOMR is required if any rise is shown. • Elevation Certificates required for applicable structures per County Floodplain Order and 44 CFR 60.3(d). | Developer / County |
| ☐ | 5 | Off-Site Drainage Easement (if applicable) • If storm water will be released to a waterway or drainageway crossing property not owned by the applicant, a recorded drainage/discharge easement from the affected owner(s) is required. • Recorded easement (or executed agreement) must be provided prior to permit issuance. | Developer |
| ☐ | 6 | Fire Marshal Office Review • Contact the Fire Marshal office at emc@sanpatriciocountytx.gov, with the Floodplain Manager (sboutwell@sanpatriciocountytx.gov) copied on the email. • Basic requirement: roads must be 20' wide and able to sustain an 88,000 lb (88k#) fire apparatus. • An entrance and exit (two points of access) are required unless an approved turnaround is provided, if the driveway exceeds 150ft. • These items must be marked on the plans. • The front of the plans must be labeled with the AHJ: San Patricio County Fire Marshal Office — Code: 2018 IFC. | Developer / Fire Marshal |
| ☐ | 7 | Plan Review & Comments • County reviews submittal for completeness and compliance; written comments issued to applicant. • Applicant revises and resubmits as needed until requirements are satisfied. | County / Developer |
| ☐ | 8 | Permit Issuance • Floodplain Development Permit and any associated permits issued once all floodplain and drainage requirements are met. | County |
| ☐ | 9 | Construction & Inspections • Periodic inspections during construction as required (Permitting & Fire). • Elevation Certificate(s) and as-built survey data submitted at appropriate construction milestones. | County / Developer |
| ☐ | 10 | Final Compliance & Close-Out • Fire Marshal office inspection must be completed. • Final documentation reviewed; Certificate of Compliance (C.O.C.) issued. • Records retained in the County floodplain file for the site. | County / Fire Marshal Office |

## Additional Notes & County Contact

Right-of-way / road use agreements: County Engineer John Hernandez — JHernandez@sanpatriciocountytx.gov. All platting is routed through the Floodplain office.

Key contacts: Floodplain Manager — sboutwell@sanpatriciocountytx.gov  |  Fire Marshal — emc@sanpatriciocountytx.gov  |  GIS/NG911 — GIS@sanpatriciocountytx.gov

Susan Boutwell, CFM, TEM — Deputy Emergency Management Coordinator / Floodplain & Permits Manager

219 W. 5th Street, Sinton, Texas  |  Phone: 361-587-3567